LITERATURE REVIEW – CFD METHODOLOGY FOR THADES

Review of Two BES-CFD Coupling Papers by Yang et al. – Relevance to the THADES Doctoral Project

Both papers are authored by Xianzhe Yang (Ph.D. candidate, Kyushu University). They are directly relevant because they solve the exact methodological problem at the core of my research proposal (RQ3): how to couple building energy simulation (BES) with high definition CFD to evaluate a hybrid passive-active system’s energy performance. Yang et al.’s Paper (Optimization of the boundary condition treatment for coupled BES-CFD simulation) explicitly identifies what my Ph.D. research proposal can adopt, hot the calculation works, and what it implies for my research scripting.

# THE CORE IDEA, IN PLAINS TERMS

Both papers address a limitation this research face. BES (e.g. EnergyPlus, THERB) is fast and good at long-term energy balance, but treats each room as one well-mixed node – it cannot see local airflow or compute an accurate convective heat transfer coefficient (CTHC). CFD sees the airflow field in detail, but is computationally expensive and cannot run ling annual periods. The solution is to couple them: CFD supplies the near-wall airflow information BES lacks, and BES supplies the surface thermal conditions CFD needs. This is precisely the bridge this research (RQ3) requires – quantifying how effectively the THADES active-passive system reduces energy demand needs both the flow field (CFD) and the energy balance (BES).

Master Degree: This research used EnergyPlus + Airflow Network. Ph.D. → These papers (Yang et al.) show the next step up: replacing the network approximation with actual CFD-derived velocities feeding the energy model. This is the methodological backbone for evaluating. The THADES energy effectiveness in Phase 3 – and it has been pre-validated.

# JOURNAL ARTICLE IDENTIFICATION

## Journal Article 1 – Application

## Title: Numerical modeling of biophilic design incorporating large-scale waterfall into a public building: Combined simulation of heat, air, and water transfer.

## Author:  X. Yang, A. Ozaki, Y. Arima, Y. Choi, S.-J. Yoo

## Journal:  Building and Environment (2024) – Q1

Demonstrates a full BES-CFD coupled model applied to a real building with waterfalls/ponds, adding a novel water-evaporation model (water potential as moisture-transfer driving force). Its value to this research is the worked example of the coupling workflow and the heat/moisture transfer treatment – the latent/moisture dimension is directly analogous to this research (incl. previous master degree thesis research topic) sensible-latent decoupling problem.

## Journal Article 2 – Method Optimization ∙ Most Relevant

## Title: Optimization of the boundary condition treatment for coupled BES-CFD simulation.

## Author:  X. Yang, S. Yoon, S.-J. Yoo, Y. Arima, Y. Choi, W. Jing, A. Ozaki

## Journal:  Building and Environment (2026) – Q1

Proposes a lightweight coupling method that minimizes CFD dependence while keeping accuracy. Its central move: rather than extracting CHTC directly from CFD (which demands very fine near-wall mesh and turbulence expertise), it takes only the CFD free-stream velocity and computes CHTC inside BES via dimensionless-number correlations. The method builds on Pappas & Zhai (2008), cited by Yang’s journal article, who applied BES-CFD coupling to a buoyancy-driven and derived CHTC correlations – the same mechanism and coupling logic central to THADES, here demonstrated on a single DSF rather than an integrated envelope.

# HOW THE CALCULATION ACTUALLY WORKS

The methodological heat is the CHTC (convective heat transfer coefficient) – the quantity that links airflow to energy. Here is the logic both papers share, in adaptable form.

The data-exchange loop

- BES computes interior surface temperature (Ts) for each wall at time step - Ts passed to CFD as thermal boundary condition - CFD computers near-wall free-stream velocity (v) at each surface - v passed back to BES - BES computes CHTC (α) from v dimensionless correlations → updates heat load & surface temps - Loop repeats each exchange step

The CHTC computation (the script)

CHTC (α) is derived from the Nusselt number, and flow regime is selected using dimensionless number. The equations below reproduced exactly as numbered in Journal Article 2 (Optimization of the boundary condition treatment for coupled BES-CFD simulation), in their original form and order.

α=Nu∙λl – CHTC from Nusselt number Nu, conductivity λ, characteristic length l

Natural Convection – when Ri > 10:

[Vertical Plane]

NuN=0.0241(Gr∙Pr)0.4						(1)

Gr=gβ∆Tl3μ2											(2)

[Horizontal Plane]

NuN=C∙Rafx									(3)

Raf=Gr∙Pr										(4)

Tf=(Ts+Troom)2										(5)

Forced Convection – when Ri < 0.1:

NuF=0.037Re0.8Pr13							(6)

Mixed Convection – combining the two:

Num=nNuNn+NuFn							(7)

Where Gr = Grashof number, Pr = Prandtl number, Ra = Rayleigh number, Re = Reynolds number; g = 9.8 m/s2; β = thermal expansion coefficient (1/K); μ = kinematic viscosity (m2/s). property values taken at film temperature Tf.

# ARTICLE 1 VS. ARTICLE 2 – WHICH TO FOLLOW FOR WHAT

| Aspect | Journal Article 1 | Journal Article 2 |
| --- | --- | --- |
| Coupling type | Static, one step (low sensitivity case) | Dynamic, sequential per time step + reduced-frequency variant |
| CHTC source | From CFD wall-adjacent velocity (0.1 m) + dimensionless eqs. | Same principle, but formalized & optimized; omits ∆Troom,a |
| Key contribution | End-to-end worked example + moisture/latent modelling | Lightweight method → less CFD cost; DSF-cavity relevance |
| Validation | Real public building, measured data | Free-running & air-conditioned; systematic BC study |

# IMPLICATION FOR THE SCRIPTING

The following section will consider the implication of the aforementioned points for the scripting process (Ph.D.). The concrete implications of the proposal for the pipeline will be considered here (Python).

- It is not necessary to extract CHTC directly from CFD. The primary efficiency of Journal Article 2 is achieved through the utilization of script CFD, which is employed to generate a specific output: the near-wall free stream velocity per surface. Subsequently, the CHTC is computed within the BES-side code by means of the dimensionless correlations. This is a far more efficient method than resolving the near-wall thermal boundary layer in CFD. It is evident that the script under consideration requires the incorporation of a regime selector. The function is to be computed per surface per time step, as follows: Ri is to be calculated → the function is to branch to natural, mixed or forced correlation; and, the result is to be returned as α. This function is both compact and testable, and thus constitutes a natural first scripting module.

- The coupling frequency is a variable can be adjusted according to user preference. Journal Article 2 demonstrates that it is possible to reduce the frequency of CFD calls in order to reduce costs; however, it is imperative to increase this frequency in the event of rapid temperature fluctuations. The build frequency should be conceptualized as a parameter, rather than a hard-coded step.

- The core of the system is data exchange orchestration. A controller script is responsible for transmitting Ts (BES→CFD) and (CFD→BES) at the point of each exchange step. This is precisely the role of a coupling manager (e.g. Python orchestrating EnergyPlus + CFD solver) – the concept being analogous to that of BCVTB/co-simulation setups referenced in Journal Article 2.

- The latent/moisture path (as delineated in Journal Article 1) is a crucial consideration in this analysis. In the context of THADES’s sensible-latent split, Journal Article 1’s moisture-transfer treatment provides a template for the management of humidity transport in conjunction with heat. This aspect is pertinent to the quantification of the latent penalty that must be addressed by active supplementation.

# STRATEGIC VALUE FOR PHD PROPOSAL

Beyond the methodological considerations, these Journal Articles serve to reinforce one’s position in two distinct ways. Firstly, a concrete, validated, supervisor-endorsed methodology is provided for RQ3, as opposed to the vague promise of a “coupled simulation.” This directly addresses the credibility issue previously identified. Secondly, the method is characterized by its ability to provide a clear framework for the contribution, with its validation being demonstrated in conventional rooms and building with waterfalls system. The application and adaptation of this method to an integrated passive – active tropical envelope (THADES) represents a novel territory, thereby reinforcing the novelty of the approach rather than competing with it.